| Student Name | G. Manasa |  |
| --- | --- | --- |
| Student Registration Number | 241U1R3007 | Class &Section: CSE-DS |
| Study Level: UG/PG | UG | Year &Term: II & I |
| Subject Name | Digital Electronics |  |

| Name of the Assessment | Group Assignment |
| --- | --- |
| Date of Submission | 09-08-2025 |

GROUP ASSIGNMENT

LG-1

1. a. Compare 1’s and 2’s complement methods with examples.

1’s Complement:

- To find the 1’s complement of a binary number, invert all the bits (change 0s to 1s and vice versa).

- Used for representing negative numbers in binary systems.

- There are two representations of zero (positive and negative zero), which can cause complications in arithmetic operations.

Example:
For 8-bit binary 01001011 (which is 75 in decimal),
1’s complement = 10110100 (simply flip each bit).

## 2’s Complement:

## To find the 2’s complement, first find the 1’s complement and then add 1 to the least significant bit (LSB).

## Most widely used method for signed number representation because it eliminates the problem of two zeros and simplifies arithmetic.

## Example:
Original: 0101 1010
1’s Complement: 1010 0101
Add 1: 1010 0101 + 1 = 1010 0110

Differences:

- 2’s complement has only one representation for zero; 1’s complement has two.

- 2’s complement arithmetic is simpler and more consistent.

b. Represent -25 in 8-bit 2’s complement and explain the steps.

Step 1: Write the positive value of 25 in binary (8 bits):
25 = 0001 1001

Step 2: Take the 1’s complement (invert all bits):
1110 0110

Step 3: Add 1 to get the 2’s complement:
1110 0110 + 1 = 1110 0111

So, -25 in 8-bit 2’s complement:
1110 0111

3. Explain the Excess-3 code and its applications.

Excess-3 Code:

- Non-weighted binary code.

- Each decimal digit is represented by its 4-bit BCD value plus 3 (i.e., “excess” of 3).

- Decimal digit ‘n’: XS-3 = BCD(n) + 0011

Example:
Decimal 5: BCD = 0101
XS-3 = 0101 + 0011 = 1000

Applications:

- Used in digital systems to simplify decimal arithmetic (e.g., adders).

- Self-complementing property allows simple error detection and simplification in subtraction.

4. Realize F = AB + A’C using basic gates and explain each step

Given: F=AB+A′CF=AB+A′C

- AND Gate: AB

- NOT Gate: A → A′ (invert A)

- AND Gate: A′C

- OR Gate: Combine (AB) + (A′C)

Step-by-step Realization:

- Inputs: A, B, C

- Through a NOT gate, get A′.

- First AND gate receives A and B → AB.

- Second AND gate receives A′ and C → A′C.

- OR gate sums outputs: (AB) + (A′C).

5. Explain the basic postulates and operations of Boolean algebra.

Postulates:

- A variable can take value 0 or 1.

- Complement: For any value A, A + A′ = 1 and AA′ = 0.

- Identity: A + 0 = A, A·1 = A.

- Domination: A + 1 = 1, A·0 = 0.

- Idempotent: A + A = A, A·A = A.

- Distributive, associative, and commutative laws apply.

Basic Operations:

- AND (·): Output 1 if both inputs are 1.

- OR (+): Output 1 if at least one input is 1.

- NOT (′): Inverts the input.

6. Perform the following decimal additions in the 8421(BCD) code

a. 25+13                b.679.6+536.8

a. 25 + 13

- 25 in BCD: 0010 0101

- 13 in BCD: 0001 0011

Add:

0010 0101

+ 0001 0011

= 0011 1000 (no correction needed, as result < 10 in each digit)

0011 1000 → 38 in decimal

b. 679.6 + 536.8

8421 BCD representation for each digit:

- 6: 0110

- 7: 0111

- 9: 1001

- . : (Decimal point; not coded in BCD)

- 6: 0110

- 5: 0101

- 3: 0011

- 6: 0110

- . : (Decimal point)

- 8: 1000

Step 1: Add Decimal Digits

- 6 (0110) + 8 (1000) = 1110 (14 decimal, invalid BCD, add 0110 for correction)

- 1110 + 0110 = 1 0100 (carry 1 to next digit, result 4 in decimal place)

Continue adding and adjusting with BCD rules for each column:

- For brevity, after correction, BCD sum:
6 7 9 . 6

- 5 3 6 . 8
= 1 2 1 6 . 4

Final BCD Result:
0001 0010 0001 0110 . 0100
Which is 1,216.4 in decimal