| Student Name | G. Manasa |  |
| --- | --- | --- |
| Student Registration Number | 241U1R3007 | Class &Section: CSE-DS |
| Study Level: UG/PG | UG | Year &Term: II & I |
| Subject Name | Digital Electronics |  |

| Name of the Assessment | Group Activity Report |
| --- | --- |
| Date of Submission | 31-08-2025 |

Group Activity Report

Topic: Combinational Logic II
Speaker: Prof. Onur Mutlu, ETH Zürich

Introduction

The expert talk Combinational Logic II by Prof. Onur Mutlu was part of the Design of Digital Circuits course at ETH Zürich, Spring 2019. The lecture presented a deep dive into combinational logic, which forms the basis of computer hardware. Unlike sequential logic, combinational logic is memoryless, meaning its outputs depend solely on current inputs, not on past history.

This lecture was especially important because it connected the lowest-level building blocks (transistors and gates) to higher-level hardware structures (adders, multiplexers, PLAs) and finally to abstract hardware design methodologies (HDLs like Verilog).

- Foundations: Transistors and CMOS Logic Gates

  - Transistors as switches: A digital system is ultimately built from billions of transistors, primarily nMOS and pMOS devices.

  - CMOS technology: Complementary MOS (CMOS) circuits pair pMOS (pull-up network) and nMOS (pull-down network) so that only one conducts at a time, ensuring efficiency.

  - Basic gates: NOT, NAND, and NOR were explained in detail, and their truth tables and CMOS implementations were shown.

  - Scalability: Larger gates with multiple inputs (e.g., 3-input AND, 10-input NOR) extend these fundamentals.

- Power consumption:

- Dynamic power = C × V² × f, where C is capacitance, V is supply voltage, and f is frequency.

- Static power = V × leakage current.

- Energy = Power × Time.

This reinforced how every logic gate in a processor ultimately comes down to transistors. I realized that circuit designers must balance speed, power, and reliability at this level. Power formulas also gave me perspective on why modern processors prioritize low voltage and why leakage is such a challenge in nanometer technologies.

2. Boolean Algebra

- Boolean algebra is the mathematical language of logic design.

- It consists of axioms, laws, and theorems that define operations on binary variables (0 and 1).

- Laws such as commutative, distributive, idempotent, and complementarity are essential for simplification.

- Duality principle: Every Boolean law has a dual (AND ↔ OR, 0 ↔ 1).

- DeMorgan’s Laws provide transformations between AND/OR with complements, making NAND and NOR equivalent to other gates.

This section highlighted the elegance of Boolean algebra. What struck me is how 19th-century mathematics by George Boole directly governs the design of 21st-century microprocessors. The concept of duality was eye-opening—it means once we prove a law, we automatically know another one.

3. Canonical Forms (SOP and POS)

- Sum of Products (SOP): Output is expressed as OR of minterms (cases where function = 1).

- Product of Sums (POS): Output is expressed as AND of maxterms (cases where function = 0).

- Minterm: AND of all variables in complemented or non-complemented form (represents one row of the truth table).

- Maxterm: OR of all variables in complemented or non-complemented form.

- Canonical forms are standardized, but not always minimal.

The canonical forms showed me how any truth table can be written algebraically. SOP feels intuitive (sum of all “true” cases), while POS complements it. I now see how these representations are essential for building programmable devices like PLAs and FPGAs.

4. Logic Simplification

- A circuit in canonical form may be correct but inefficient. Simplification reduces hardware cost.

- Uniting Theorem: If two terms differ by only one variable, that variable can be eliminated.

- Karnaugh Maps (K-Maps): Visual truth tables that help group adjacent 1s (or 0s) into rectangles of size 2^k, eliminating variables.

- Don’t Care conditions: Unused input combinations can be treated as 0 or 1 to simplify circuits further.

This section was one of the most useful. K-Maps turned simplification into a geometric puzzle. Grouping 1s felt intuitive, and I realized that EDA tools today are essentially automating this process for circuits with many variables.

5. Combinational Building Blocks

Prof. Mutlu introduced standard components built from logic gates:

- Decoder: Converts binary inputs into 1-out-of-2ⁿ outputs. Example: Instruction decoding in CPUs.

- Multiplexer (MUX): Routes one of many inputs to a single output based on select lines. Essential in data routing.

- Full Adder: Performs binary addition with carry. Multi-bit adders (e.g., ripple-carry adder) are built from 1-bit adders.

- Programmable Logic Array (PLA): Implements arbitrary logic using a regular array of AND-OR gates.

- Tri-State Buffer: Allows multiple devices to share a bus by putting outputs in high-impedance (Z) state when inactive.

This section gave me insight into how small building blocks scale up into entire CPUs. For example, decoders reminded me of DRAM row addressing, while MUXes reminded me of ALU input selection. The PLA discussion linked back to SOP forms and showed the importance of programmability.

6. Logic Completeness

- A set of gates is functionally complete if it can implement any Boolean function.

- {AND, OR, NOT} is complete.

- NAND alone or NOR alone is also complete.

I found this both surprising and fascinating: one single type of gate (NAND or NOR) can build an entire computer. It showed me the power of universality in logic design.

7. Hardware Description Languages (HDLs) – Verilog

- As circuits grew to billions of transistors, manual design became impossible.

- HDLs like Verilog and VHDL allow designers to describe, simulate, and synthesize hardware.

- Structural modeling: Writing modules in terms of gates and interconnections.

- Behavioral modeling: Writing higher-level functional equations (e.g., assign y = a & b).

- Design methodologies:

  - Top-down (start with system, break into modules).

  - Bottom-up (start with building blocks, assemble system).

- Verilog syntax for modules, inputs/outputs, buses, operators, conditional assignments, etc.

This was perhaps the most practical part of the lecture. I understood why HDLs are the industry standard—they allow abstraction and scalability. I was inspired to think of Verilog as the “programming language of hardware”, bridging my coding skills with circuit design.

The lecture tied together many layers of digital design:

- From physics of transistors → logic gates → Boolean algebra → simplification → building blocks → complete systems with HDLs.

- It reinforced that computer architecture is not magic but the systematic layering of abstractions.

- I also connected the lecture to real-world challenges:

  - Moore’s Law slowing → need for optimization.

  - Power constraints in mobile devices → importance of simplification.

  - Complex designs like Intel’s Kaby Lake → necessity of HDLs.

What I admired most was Prof. Mutlu’s clear progression from low-level basics to high-level methodologies. It made me realize that being a good hardware engineer requires not just understanding theory, but also appreciating scalability, power, and design automation.

Conclusion

The expert talk on Combinational Logic II was both foundational and forward-looking. It helped me strengthen my basics of transistors and Boolean algebra, while also exposing me to advanced tools like Verilog.

My key personal takeaways are:

- Every modern computer is grounded in Boolean algebra.

- Simplification matters as much as correctness—an optimized design saves power and cost.

- Abstraction is essential—without HDLs, modern chip design would be impossible.

This lecture not only deepened my knowledge but also motivated me to practice logic simplification, master K-Maps, and begin hands-on learning with Verilog. I now have a clearer vision of how combinational logic underpins the architecture of the future.