| Name of the Student: G. Manasa | Academic Year: 2024-25 |
| --- | --- |
| Student Registration Number: 241U1R3007 | Year & Term: II&I |
| Study Level: UG | Class & Section: CSE-DS-2A |
| Name of the Course: Digital Electronics | Name of the Instructor: Nuzhath Farhana mam |
| Name of the Assessment: Lab-Report-8 | Date of Submission: 31-08-2025 |

Lab-Report 8

## Simulation of Flip-Flops Using NAND and NOR Gates

## Simulation of SR Flip-Flop Using NAND and NOR Gates

## 1.Objectives

- To design and simulate an SR flip-flop using NAND and NOR gates in SimulIDE.

- To verify the set, reset, hold, and invalid conditions.

- To understand the truth table and working of SR flip-flops.

- To analyse the difference between active-high (NOR) and active-low (NAND) SR flip-flops.

### 2.Software Required:

PC with SimulIDE software installed

## 3.Procedure

### A) SR Flip-Flop using NOR Gates:

- Place 2 NOR gates, 2 switches for inputs (S and R), 2 LEDs for outputs (Q, \bar{Q}), VCC, and GND.

- Connect the output of NOR1 to input of NOR2, and output of NOR2 to input of NOR1 (cross-coupling).

- Apply the input combinations and observe the outputs using LEDs.

### B) SR Flip-Flop using NAND Gates:

- Place 2 NAND gates, 2 switches for inputs (\bar{S}, \bar{R}), 2 LEDs for outputs (Q, \bar{Q}), VCC, and GND.

- Cross-couple outputs as in NOR version.

- Apply active-low inputs and verify the results.

### 4.Circuit Diagrams:

### a.SR Flip-Flop Using NAND Gates

### In a SR flip flop using NAND gates, two NAND gates are connected in a feedback loop. The inputs are S (Set) and R (Reset), and the outputs are Q and Q'.

#### Truth Table for NAND-based SR Flip-Flop:

| S | R | Q | Q' |
| --- | --- | --- | --- |
| 0 | 0 | Previous State | Previous State (inverted) |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | Invalid State | Invalid State |

Simulation:

b. SR Flip-Flop Using NOR Gates

In a SR flip-flop using NOR gates, two NOR gates are connected in a feedback loop. The inputs are S (Set) and R (Reset), and the outputs are Q and Q'.

Truth Table for NOR-based SR Flip-Flop:

| S | R | Q | Q' |
| --- | --- | --- | --- |
| 0 | 0 | Previous State | Previous State (inverted) |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | Invalid State | Invalid State |

Simulation:

5.Result:

The SR flip-flop was designed, implemented, and tested using NAND and NOR gates. The simulation verified correct functionality and demonstrated how cross-coupled gates form a basic bistable storage element.

Simulation of JK Flip-Flop Using NAND and NOR Gates

1.Objectives

- To design and simulate a JK flip-flop using NAND and NOR gates in SimulIDE.

- To verify set, reset, hold, and toggle operations.

- To understand the truth table and working of JK flip-flops.

- To analyze how JK flip-flops eliminate the invalid state of SR flip-flops.

2.Software Required:

PC with SimulIDE software installed

3.Theory

The JK flip-flop is an improvement over the SR flip-flop. It removes the invalid state by allowing toggling when both inputs are active. It can be implemented using cross-coupled NAND or NOR gates with additional feedback paths.

1) JK Flip-Flop Operation:

- Inputs: J (Set), K (Reset), CLK (Clock)

- Outputs: Q (Normal), \bar{Q} (Complement)

- Operation:

  - J=0, K=0 → Hold (No change)

  - J=0, K=1 → Reset Q=0

  - J=1, K=0 → Set Q=1

  - J=1, K=1 → Toggle (Q switches state)

2) JK Flip-Flop Using NAND Gates:

- Implemented with 4 NAND gates and feedback connections.

- Edge-triggered version uses additional clock gating NAND gates.

3) JK Flip-Flop Using NOR Gates:

- Less common but possible by modifying SR NOR latch with feedback.

4.Procedure

A) JK Flip-Flop using NAND Gates:

- Place 4 NAND gates (plus 2 for clock if edge-triggered), 2 switches for J and K, 1 switch for CLK, 2 LEDs for Q and \bar{Q}.

- Cross-couple outputs and connect feedback paths.

- Apply clock pulses and various J, K inputs. Observe outputs.

B) JK Flip-Flop using NOR Gates:

- Place 4 NOR gates, J, K, CLK switches, LEDs for Q and \bar{Q}.

- Connect according to JK logic with feedback.

- Apply inputs and verify truth table.

5.Circuit Diagrams:

a. JK Flip-Flop Using NAND Gates

Truth Table

| Clk | J | K | Q | Description |
| --- | --- | --- | --- | --- |
| 0 | X | X | Q | Clk in 0 no change in Q |
| 1 | 0 | 0 | Q | Memory (no change) |
| 1 | 1 | 0 | 1 | Set |
| 1 | 0 | 1 | 0 | Reset |
| 1 | 1 | 1 | Q̅ | Toggle |

Simulation:

b.JK Flip-Flop Using NOR Gates

Simulation:

6.Result:

The JK flip-flop was designed and simulated using NAND and NOR gates in SimulIDE. The experiment verified set, reset, hold, and toggle operations, showing how the JK flip-flop improves upon the SR flip-flop

Simulation of D Flip-Flop Using NAND and NOR Gates

1.Objectives

- To design and simulate a D flip-flop using NAND and NOR gates in SimulIDE.

- To verify data storage and clocked operation of a D flip-flop.

- To understand the truth table and working of a D flip-flop.

2.Software Required:

PC with SimulIDE software installed

3.Theory

The D flip-flop (Data or Delay flip-flop) is derived from the SR flip-flop by eliminating the invalid state. It has a single input D and a clock input CLK.

1) D Flip-Flop Operation:

- Inputs: D (Data), CLK (Clock)

- Outputs: Q (Normal), \bar{Q} (Complement)

- Operation:

  - On the rising (or falling) edge of CLK, Q follows D.

  - When CLK is inactive, Q holds the previous state.

2) D Flip-Flop Using NAND Gates:

- Built from SR NAND latch with an inverter ensuring S=\bar{D} and R=D, controlled by CLK.

- Requires 4 NAND gates (plus 2 for clock control).

3) D Flip-Flop Using NOR Gates:

- Constructed similarly with an SR NOR latch and input gating.

4.Procedure

A) D Flip-Flop using NAND Gates:

- Place NAND gates, switch for D, switch for CLK, and LEDs for Q and \bar{Q}.

- Connect the circuit as per D flip-flop logic with clock gating.

- Apply various D values and clock pulses, observe Q output.

B) D Flip-Flop using NOR Gates:

- Place NOR gates for latch and input gating.

- Apply input values and clock signals to verify Q follows D on clock edge.

5.Circuit Diagrams:

a.D Flip Flop Using NAND Gates

Simulation:

b.D Flip Flop Using NOR Gates

Simulation:

6.Result:

The D flip-flop was designed and simulated using NAND and NOR gates in SimulIDE. It successfully demonstrated data storage synchronized with clock pulses and eliminated the invalid state of SR flip-flops.

Simulation of T Flip-Flop Using NAND and NOR Gates

## 1.Objectives

- To design and simulate a T flip-flop using NAND and NOR gates in SimulIDE.

- To verify the toggle operation of a T flip-flop.

- To understand the truth table and applications of a T flip-flop.

### 2.Software Required:

PC with SimulIDE software installed

## 3.Theory

The T (Toggle) flip-flop is derived from the JK flip-flop by connecting J = K = T. It changes its output state (toggles) on every active clock edge when T=1, and holds its state when T=0.

### 1) T Flip-Flop Operation:

- Inputs: T (Toggle), CLK (Clock)

- Outputs: Q (Normal), \bar{Q} (Complement)

- Operation:

  - T = 0: Q remains unchanged (Hold)

  - T = 1: Q toggles on each clock edge (0→1 or 1→0)

### 2) T Flip-Flop Using NAND Gates:

- Implemented from a JK NAND flip-flop with J and K tied together.

- Requires 4 NAND gates for the basic JK latch and additional gates for clock control.

### 3) T Flip-Flop Using NOR Gates:

- Implemented similarly using a JK NOR latch with tied inputs.

## 4.Procedure

### A) T Flip-Flop using NAND Gates:

- Place NAND gates, switch for T, switch for CLK, and LEDs for Q and \bar{Q}.

- Connect the circuit as per T flip-flop logic with toggle operation.

- Apply T=1 and clock pulses, observe toggling of Q

### B) T Flip-Flop using NOR Gates:

- Place NOR gates for the latch and clock gating.

- Apply T=1, provide clock pulses, verify Q toggles each cycle.

5.Circuit Diagrams:

a. T Flip Flop Using NAND Gates

Simulation:

b.T Flip Flop Using NOR Gates

Simulation:

6.Result:

The T Flip-Flop was successfully implemented in SimulIDE using NAND and NOR gates. The output Q toggled on each active clock edge when T=1, confirming correct operation.