| Name of the Student: G. Manasa | Academic Year: |
| --- | --- |
| Student Registration Number: 241U1R3007 | Year & Term: |
| Study Level: B-Tech | Class & Section: CSE-DS |
| Name of the Course: Digital Electronics | Name of the Instructor: |
| Name of the Assessment: lab report 10 | Date of Submission: 17-08-2025 |

Simulation of Asynchronous and Synchronous Counters

Simulation of Asynchronous Counters using JK Flip-Flops

1.Objective

- Design and simulate a 4-bit asynchronous (ripple) binary up counter using JK flip-flops in Logisim.

- Design and simulate a 4-bit asynchronous down counter using JK flip-flops in Logisim.

- Design and simulate a 4-bit asynchronous up-down counter using JK flip-flops in Logisim.

- Observe and record the counting sequence and verify expected outputs.

2.Apparatus Required:

PC with Logisim installed.

3.Theory

Asynchronous (ripple) counters operate by clocking only the first flip-flop with the external clock, while subsequent flip-flops are clocked by outputs of the preceding flip-flops. JK flip-flops can toggle when both inputs J=1 and K=1, which makes them highly suitable for counter design.

JK Flip-Flop Characteristics

- Inputs: J, K, CLK, Q, Q̄

- Function: When J=K=1, Q toggles on every active clock edge.

- For counters, tie J=K=1 to force toggling on each clock.

4.Procedure

- Create new project async_jk_counters.circ.

- Build 4-bit up counter, run and record outputs.

- Build 4-bit down counter, run and record outputs.

- Build 4-bit up-down counter with mode control.

- Record observations.

5.Design and Implementation

A. 4-bit Asynchronous Up Counter

Each JK flip-flop toggles by setting J=K=1. First flip-flop is driven by external clock, others by the Q output of previous stage.

Steps:

- Place 4 JK flip-flops. Label outputs Q0 (LSB), Q1, Q2, Q3 (MSB).

- Tie J=K=1 for all flip-flops.

- Connect external clock → CLK of Q0.

- Q0 → CLK of Q1, Q1 → CLK of Q2, Q2 → CLK of Q3.

- Connect outputs Q0–Q3 to LEDs.

Counts 0 → 15 → repeat

| Clock Pulse | Q3 | Q2 | Q1 | Q0 | Decimal |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 2 | 0 | 0 | 1 | 0 | 2 |
| 3 | 0 | 0 | 1 | 1 | 3 |
| 4 | 0 | 1 | 0 | 0 | 4 |
| 5 | 0 | 1 | 0 | 1 | 5 |
| 6 | 0 | 1 | 1 | 0 | 6 |
| 7 | 0 | 1 | 1 | 1 | 7 |
| 8 | 1 | 0 | 0 | 0 | 8 |
| 9 | 1 | 0 | 0 | 1 | 9 |
| 10 | 1 | 0 | 1 | 0 | 10 |
| 11 | 1 | 0 | 1 | 1 | 11 |
| 12 | 1 | 1 | 0 | 0 | 12 |
| 13 | 1 | 1 | 0 | 1 | 13 |
| 14 | 1 | 1 | 1 | 0 | 14 |
| 15 | 1 | 1 | 1 | 1 | 15 |
| 16 → | 0 | 0 | 0 | 0 | 0 |

Simulation:

B. 4-bit Asynchronous Down Counter

Similar to up counter, but use Q̄ outputs to drive the next stage, producing reverse sequence.

Steps:

- Place 4 JK flip-flops with J=K=1.

- Connect external clock → CLK of Q0.

- Use Q̄0 → CLK of Q1, Q̄1 → CLK of Q2, Q̄2 → CLK of Q3.

- LEDs on Q outputs.

Counts 15 → 0 → repeat.

| Clock Pulse; Q3; Q2; Q1; Q0; Decimal. 0; 1; 1; 1; 1; 15. 1; 1; 1; 1; 0; 14. 2; 1; 1; 0; 1; 13. 3; 1; 1; 0; 0; 12. 4; 1; 0; 1; 1; 11. 5; 1; 0; 1; 0; 10. 6; 1; 0; 0; 1; 9. 7; 1; 0; 0; 0; 8. 8; 0; 1; 1; 1; 7. 9; 0; 1; 1; 0; 6. 10; 0; 1; 0; 1; 5. 11; 0; 1; 0; 0; 4. 12; 0; 0; 1; 1; 3. 13; 0; 0; 1; 0; 2. 14; 0; 0; 0; 1; 1. 15; 0; 0; 0; 0; 0. 16 →; 1; 1; 1; 1; 15. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Simulation: |  |  |  |  |  |

C. 4-bit Asynchronous Up-Down Counter

Allow selection of up or down mode with a control input (UP/DOWN). Use multiplexing logic to decide whether to feed Q or Q̄ of each stage as the clock input to the next.

Steps:

- Place 4 JK flip-flops with J=K=1.

- Add a control input M (Mode: 0=Up, 1=Down).

- For each stage, use a multiplexer (MUX) to select between Q and Q̄ as the clock input to the next flip-flop:

  - If M=0 → Q feeds next clock (Up).

  - If M=1 → Q̄ feeds next clock (Down).

- Connect LEDs to Q outputs.

Case 1: UP Counting (M = 1)

| Clock Pulse | Q3 | Q2 | Q1 | Q0 | Decimal |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 2 | 0 | 0 | 1 | 0 | 2 |
| 3 | 0 | 0 | 1 | 1 | 3 |
| 4 | 0 | 1 | 0 | 0 | 4 |
| 5 | 0 | 1 | 0 | 1 | 5 |
| 6 | 0 | 1 | 1 | 0 | 6 |
| 7 | 0 | 1 | 1 | 1 | 7 |
| 8 | 1 | 0 | 0 | 0 | 8 |
| 9 | 1 | 0 | 0 | 1 | 9 |
| 10 | 1 | 0 | 1 | 0 | 10 |
| 11 | 1 | 0 | 1 | 1 | 11 |
| 12 | 1 | 1 | 0 | 0 | 12 |
| 13 | 1 | 1 | 0 | 1 | 13 |
| 14 | 1 | 1 | 1 | 0 | 14 |
| 15 | 1 | 1 | 1 | 1 | 15 |
| 16 → | 0 | 0 | 0 | 0 | 0 (Repeat) |

Case 2: DOWN Counting (M = 0)

| Clock Pulse | Q3 | Q2 | Q1 | Q0 | Decimal |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 15 |
| 1 | 1 | 1 | 1 | 0 | 14 |
| 2 | 1 | 1 | 0 | 1 | 13 |
| 3 | 1 | 1 | 0 | 0 | 12 |
| 4 | 1 | 0 | 1 | 1 | 11 |
| 5 | 1 | 0 | 1 | 0 | 10 |
| 6 | 1 | 0 | 0 | 1 | 9 |
| 7 | 1 | 0 | 0 | 0 | 8 |
| 8 | 0 | 1 | 1 | 1 | 7 |
| 9 | 0 | 1 | 1 | 0 | 6 |
| 10 | 0 | 1 | 0 | 1 | 5 |
| 11 | 0 | 1 | 0 | 0 | 4 |
| 12 | 0 | 0 | 1 | 1 | 3 |
| 13 | 0 | 0 | 1 | 0 | 2 |
| 14 | 0 | 0 | 0 | 1 | 1 |
| 15 | 0 | 0 | 0 | 0 | 0 |
| 16 → | 1 | 1 | 1 | 1 | 15 (Repeat) |

6.Result

- Successfully designed and simulated 4-bit asynchronous up, down, up-down counters using JK flip-flops in Logisim.

- Verified correct counting sequences.

Simulation of Synchronous Counters using JK Flip-Flops

1.Objective

- Design and simulate a 4-bit synchronous up counter using JK flip-flops in Logisim.

- Design and simulate a 4-bit synchronous down counter using JK flip-flops in Logisim.

- Design and simulate a 4-bit synchronous up/down counter using JK flip-flops in Logisim.

- Verify counting sequences and understand synchronous operation advantages over asynchronous.

2.Apparatus Required:

PC with Logisim installed.

3.Theory

Counters are sequential circuits that progress through a sequence of states depending on clock pulses. Unlike asynchronous (ripple) counters, synchronous counters use a common clock input for all flip-flops, ensuring simultaneous state changes and avoiding ripple delays.

JK Flip-Flop Operation (when J=K=1):

- Toggles its state (Q → ¬Q) on each active clock edge.

- Controlled toggling can be achieved by enabling inputs with logic expressions depending on lower-order outputs.

4-bit Synchronous Counter Design:

- Up Counter: Each higher-order flip-flop toggles when all lower-order outputs are HIGH.

- Down Counter: Each higher-order flip-flop toggles when all lower-order outputs are LOW.

- Up/Down Counter: Mode control signal (M) selects between Up and Down logic conditions.

4.Procedure

- Open Logisim and create a new project sync_jk_counters.circ.

- Implement the 4-bit synchronous up counter using JK flip-flops and logic gates.

- Verify the counting sequence by applying a clock.

- Repeat for the down counter using modified logic.

- Implement the up/down counter using mode input M.

- Test circuits and note outputs.

5.Design and Implementation

A. 4-bit Synchronous Up Counter

Logic design:

- J0 = K0 = 1 (LSB toggles every clock pulse).

- J1 = K1 = Q0.

- J2 = K2 = Q0·Q1.

- J3 = K3 = Q0·Q1·Q2.

| Clock Pulse | Q3 | Q2 | Q1 | Q0 | Decimal |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 2 | 0 | 0 | 1 | 0 | 2 |
| 3 | 0 | 0 | 1 | 1 | 3 |
| 4 | 0 | 1 | 0 | 0 | 4 |
| 5 | 0 | 1 | 0 | 1 | 5 |
| 6 | 0 | 1 | 1 | 0 | 6 |
| 7 | 0 | 1 | 1 | 1 | 7 |
| 8 | 1 | 0 | 0 | 0 | 8 |
| 9 | 1 | 0 | 0 | 1 | 9 |
| 10 | 1 | 0 | 1 | 0 | 10 |
| 11 | 1 | 0 | 1 | 1 | 11 |
| 12 | 1 | 1 | 0 | 0 | 12 |
| 13 | 1 | 1 | 0 | 1 | 13 |
| 14 | 1 | 1 | 1 | 0 | 14 |
| 15 | 1 | 1 | 1 | 1 | 15 |
| 16 → resets | 0 | 0 | 0 | 0 | 0 |

B. 4-bit Synchronous Down Counter

Logic design:

- J0 = K0 = 1.

- J1 = K1 = Q̄0.

- J2 = K2 = Q̄0·Q̄1.

- J3 = K3 = Q̄0·Q̄1·Q̄2.

| Clock Pulse | Q3 | Q2 | Q1 | Q0 | Decimal |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 15 |
| 1 | 1 | 1 | 1 | 0 | 14 |
| 2 | 1 | 1 | 0 | 1 | 13 |
| 3 | 1 | 1 | 0 | 0 | 12 |
| 4 | 1 | 0 | 1 | 1 | 11 |
| 5 | 1 | 0 | 1 | 0 | 10 |
| 6 | 1 | 0 | 0 | 1 | 9 |
| 7 | 1 | 0 | 0 | 0 | 8 |
| 8 | 0 | 1 | 1 | 1 | 7 |
| 9 | 0 | 1 | 1 | 0 | 6 |
| 10 | 0 | 1 | 0 | 1 | 5 |
| 11 | 0 | 1 | 0 | 0 | 4 |
| 12 | 0 | 0 | 1 | 1 | 3 |
| 13 | 0 | 0 | 1 | 0 | 2 |
| 14 | 0 | 0 | 0 | 1 | 1 |
| 15 | 0 | 0 | 0 | 0 | 0 |
| 16 → resets | 1 | 1 | 1 | 1 | 15 |

C. 4-bit Synchronous Up/Down Counter

Logic design (using Mode input M):

- J0 = K0 = 1.

- J1 = K1 = (M·Q0) + (M̄·Q̄0).

- J2 = K2 = (M·Q0·Q1) + (M̄·Q̄0·Q̄1).

- J3 = K3 = (M·Q0·Q1·Q2) + (M̄·Q̄0·Q̄1·Q̄2).

| Clock Pulse | M | Q3 | Q2 | Q1 | Q0 | Decimal |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 2 | 1 | 0 | 0 | 1 | 0 | 2 |
| 3 | 1 | 0 | 0 | 1 | 1 | 3 |
| 4 | 1 | 0 | 1 | 0 | 0 | 4 |
| 5 | 1 | 0 | 1 | 0 | 1 | 5 |
| 6 | 1 | 0 | 1 | 1 | 0 | 6 |
| 7 | 1 | 0 | 1 | 1 | 1 | 7 |
| 8 | 1 | 1 | 0 | 0 | 0 | 8 |
| 9 | 1 | 1 | 0 | 0 | 1 | 9 |
| 10 | 1 | 1 | 0 | 1 | 0 | 10 |
| 11 | 1 | 1 | 0 | 1 | 1 | 11 |
| 12 | 1 | 1 | 1 | 0 | 0 | 12 |
| 13 | 1 | 1 | 1 | 0 | 1 | 13 |
| 14 | 1 | 1 | 1 | 1 | 0 | 14 |
| 15 | 1 | 1 | 1 | 1 | 1 | 15 |
| 16 → resets | 1 | 0 | 0 | 0 | 0 | 0 |

| Clock Pulse | M | Q3 | Q2 | Q1 | Q0 | Decimal |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 1 | 1 | 15 |
| 1 | 0 | 1 | 1 | 1 | 0 | 14 |
| 2 | 0 | 1 | 1 | 0 | 1 | 13 |
| 3 | 0 | 1 | 1 | 0 | 0 | 12 |
| 4 | 0 | 1 | 0 | 1 | 1 | 11 |
| 5 | 0 | 1 | 0 | 1 | 0 | 10 |
| 6 | 0 | 1 | 0 | 0 | 1 | 9 |
| 7 | 0 | 1 | 0 | 0 | 0 | 8 |
| 8 | 0 | 0 | 1 | 1 | 1 | 7 |
| 9 | 0 | 0 | 1 | 1 | 0 | 6 |
| 10 | 0 | 0 | 1 | 0 | 1 | 5 |
| 11 | 0 | 0 | 1 | 0 | 0 | 4 |
| 12 | 0 | 0 | 0 | 1 | 1 | 3 |
| 13 | 0 | 0 | 0 | 1 | 0 | 2 |
| 14 | 0 | 0 | 0 | 0 | 1 | 1 |
| 15 | 0 | 0 | 0 | 0 | 0 | 0 |
| 16 → resets | 0 | 1 | 1 | 1 | 1 | 15 |

6.Conclusion

- Successfully designed and simulated 4-bit synchronous up, down, and up/down counters using JK flip-flops in Logisim.

- Verified correct counting sequences.

- Observed that synchronous counters are faster and more reliable compared to asynchronous counters.